VILLA CONSTRUCTION QUOTATION

AA CLASS RESIDENTIAL VILLA CONSTRUCTION

| Quotation Particular | Details |
| --- | --- |
| Quotation For | Villa Construction |
| Construction Category | AA Class Construction |
| Construction Rate | ₹4,000/- per sq. ft. |
| Purpose | Residential Villa Construction |
| Quotation Status | Indicative / Representational Quotation |

| Contact Person Ayush Mittal | Contact No. +91-9958157168 | Website www.willow-estates-rishikesh.com | Email info@willow-estates-rishikesh.com |
| --- | --- | --- | --- |

Important Note: This quotation is representational and is subject to actual site conditions, approved architectural and structural drawings, final material selections, specifications and mutually agreed scope of work.

1. STRUCTURAL & CIVIL WORKS

| Item | Proposed Specification |
| --- | --- |
| Structural System | Seismic-resistant RCC framed structure as per approved structural design. |
| RCC | M25 grade or equivalent, as specified by structural design. |
| PCC | 1:4:8 or as required by structural/site conditions. |
| Reinforcement Steel | Tehri Gold / Kamdhenu / equivalent approved quality. |
| Brickwork | First-class brickwork. |
| Floor-to-Floor Height | As per approved architectural drawings. |
| Roof | Part-sloped roof with steel framework wherever specified in drawings. |
| Reinforcement | As per approved structural drawings and design calculations. |

2. FLOORING & TILING

| Area | Tile / Finish | Indicative Allowance |
| --- | --- | --- |
| Staff Room | 2' × 2' vitrified tiles | ₹40–45 / sq. ft. |
| Toilet / Pantry / Washing | 1' × 1' tiles | ₹50–55 / sq. ft. |
| Parking | 1' × 1' tiles | ₹55–60 / sq. ft. |
| Toilet / Pantry Walls | 1' × 1.5' wall tiles | ₹60–65 / sq. ft. |
| Kitchen | 4' × 2' / 2' × 2' tiles | ₹65–70 / sq. ft. |
| Drawing / Dining | 4' × 2' tiles | ₹90–100 / sq. ft. |
| Balcony / Toilet | 2' × 2' tiles | ₹50–60 / sq. ft. |
| Staircase | Leather-finish granite | Approx. ₹200 / sq. ft. |
| Bedrooms | Wooden flooring | ₹100–120 / sq. ft. |

Preferred brands: Kajaria / Somany / Johnson / equivalent. Any selection above the stated allowance will be charged additionally.

3. DOORS, WINDOWS & WOODWORK

| Component | Specification |
| --- | --- |
| Door Frames | Salwood / Indian Salwood or Malaysia Red Salwood. |
| Main Entrance Door | Teakwood, design and detailing as per approved drawings. |
| Internal Doors | Shutters and frames as per approved design and specifications. |
| Door Hardware | Dorset / Jolly / equivalent approved quality. |
| Windows | Frames and shutters as per approved architectural design/specification. |
| Wardrobes | Modular wardrobes as per approved design using Greenply / equivalent quality board/material. |

4. KITCHEN

| Component | Specification / Allowance |
| --- | --- |
| Countertop | Granite countertop, approx. ₹250 / sq. ft. |
| Modular Kitchen | Modular kitchen as per approved design; cabinets, shutters and hardware as finalized. |
| Kitchen Wall Tiles | 4' × 2' / 2' × 2' tiles, approx. ₹65–70 / sq. ft. |
| Premium Accessories | Premium hardware/accessories and appliances are excluded unless specifically included in the final BOQ. |
| Appliances | Chimney, hob, dishwasher, refrigerator and other electrical appliances excluded unless specifically included. |

5. WALL FINISHING & PAINTING

| Area | Specification |
| --- | --- |
| Internal Walls | POP punning / wall preparation as required, followed by acrylic emulsion. |
| External Walls | Exterior-grade, weather-resistant paint. |
| Paint Brands | Asian Paints / Nerolac / JK Birla / equivalent. |
| Colour Selection | Client to select standard colour combinations from approved options. |

6. FALSE CEILING

False ceiling is proposed for drawing, dining and all bedrooms.

Design and finish shall be as per approved drawings/design.

Decorative, specialized or premium false-ceiling treatments beyond the standard scope may be charged additionally.

7. STAIRCASE & RAILINGS

| Component | Specification |
| --- | --- |
| Staircase Structure | MS structural frame with granite finish. |
| Staircase Finish | Leather-finish granite, approx. ₹200 / sq. ft. |
| Staircase Railing | SS top rail / balustrades using 304-grade stainless steel. |
| Balcony Railing | 8 mm toughened glass. |
| Other Railings | MS railing with enamel paint wherever specified. |

8. ELECTRICAL WORKS

| Component | Specification |
| --- | --- |
| Wiring & Conduits | Polycab / Havells / equivalent. |
| Switches & Sockets | Havells Reo / Anchor Penta / equivalent. |
| Lighting Points | As per approved electrical drawings. |
| Electrical Fittings | Havells / Polycab / equivalent. |
| Point Schedule | Detailed electrical point schedule to be finalized with approved drawings. |
| Excluded unless BOQ | Decorative lighting, chandeliers, designer fixtures, smart-home systems, inverter, solar and appliances. |

9. SANITARY & PLUMBING WORKS

| Component | Specification |
| --- | --- |
| Plumbing Pipes | CPVC using Supreme / Astral / Finolex / equivalent. |
| Sanitary Chinaware | Jaquar / Cera / equivalent. |
| CP Fittings | Jaquar / Cera / equivalent. |
| Overhead Water Tank | 1,000 litres, Sintex / Himgiri / equivalent. |
| Bathroom Fixtures | Standard fixtures as per approved design. |
| Premium Fixtures | Shower panels, bathtubs, jacuzzi, designer basins and specialized fittings extra unless specifically included. |

10. CORE MATERIAL & BRAND SCHEDULE

| Material / Work | Preferred Brand / Specification |
| --- | --- |
| Cement | Ambuja / UltraTech / JK Super / equivalent |
| Reinforcement Steel | Tehri Gold / Kamdhenu / equivalent |
| Bricks | First Class |
| Structural RCC | M25 / equivalent |
| Tiles | Kajaria / Somany / Johnson / equivalent |
| Plumbing | Supreme / Astral / Finolex / equivalent |
| Electrical Wire | Havells / Polycab / equivalent |
| Paint | Asian Paints / Nerolac / JK Birla / equivalent |
| Sanitaryware | Jaquar / Cera / equivalent |
| CP Fittings | Jaquar / Cera / equivalent |
| Door Hardware | Dorset / Jolly / equivalent |

Equivalent brands may be used subject to availability and prior approval.

11. EXTERNAL DEVELOPMENT & OTHER WORKS

| Item | Scope / Remarks |
| --- | --- |
| Main Gate | As per approved design. |
| Boundary Wall | First-class brickwork, plaster and standard finish; exclusive/decorative treatment extra. |
| Biodigester / Septic Tank | For approx. 8–10 users; additional cost may apply depending on design, capacity and site conditions. |
| Soak / Recharge Pit | As required by site conditions; cost depends on soil condition, size and design. |

12. EXTRA / ADDITIONAL WORKS

| S. No. | Additional / Chargeable Item |
| --- | --- |
| 1 | Additional wall, structural modification or construction not shown in approved drawings. |
| 2 | Damage to existing property/structures caused by additional or alteration work. |
| 3 | Additional excavation due to unforeseen site conditions. |
| 4 | Rock excavation or difficult soil conditions. |
| 5 | Retaining walls. |
| 6 | Decorative or premium boundary wall treatments. |
| 7 | Premium tiles, marble, granite or wood above the stated allowance. |
| 8 | Premium sanitaryware or CP fittings above the stated range. |
| 9 | Additional electrical points beyond the approved point schedule. |
| 10 | Home automation / smart-home systems. |
| 11 | Solar systems. |
| 12 | DG set / inverter systems. |
| 13 | Air-conditioning equipment and installation. |
| 14 | Kitchen appliances. |
| 15 | Loose furniture. |
| 16 | Curtains / blinds. |
| 17 | Landscaping or specialized horticulture. |
| 18 | Water bodies, fountains or swimming pools unless specifically included. |
| 19 | Changes requested after approval of drawings. |
| 20 | Any item specifically marked 'Extra' in the final BOQ. |

13. CONSTRUCTION RATE & ILLUSTRATIVE PROJECT COST

Revised AA Class Construction Rate: ₹4,000/- per sq. ft.

| Illustrative Construction Area | Rate | Estimated Construction Cost |
| --- | --- | --- |
| 1,500 sq. ft. | ₹4,000 / sq. ft. | ₹60,00,000 (₹60 Lakh) |
| 2,000 sq. ft. | ₹4,000 / sq. ft. | ₹80,00,000 (₹80 Lakh) |
| 2,500 sq. ft. | ₹4,000 / sq. ft. | ₹1,00,00,000 (₹1 Crore) |
| 3,000 sq. ft. | ₹4,000 / sq. ft. | ₹1,20,00,000 (₹1.20 Crore) |

Final project value shall be calculated on the actual approved construction area and the mutually agreed final BOQ/scope.

14. COMMERCIAL TERMS & CONDITIONS

| S. No. | Commercial Term / Condition |
| --- | --- |
| 1 | Quotation is based on the specifications and approved architectural/structural drawings. |
| 2 | The quoted construction rate is revised to ₹4,000/- per sq. ft., subject to the stated scope and specifications. |
| 3 | Any change in design, specifications, materials or scope shall be evaluated and priced separately. |
| 4 | Premium materials selected above the stated allowance shall be charged at the actual differential cost. |
| 5 | Government fees, statutory charges, approval fees, development charges and authority charges shall be borne by the client unless specifically included. |
| 6 | Electricity and water connection charges, utility deposits and related authority charges are excluded unless specifically included. |
| 7 | Soil testing and specialized structural requirements arising from site conditions shall be charged separately where applicable. |
| 8 | Unforeseen site conditions requiring additional work shall be treated as extra work. |
| 9 | The final mutually approved and signed BOQ/material specification shall prevail over this indicative quotation. |
| 10 | Construction shall commence after approval of drawings, site handover and completion of the agreed payment schedule/formalities. |
| 11 | Client shall approve relevant samples, finishes and material selections before procurement wherever required. |
| 12 | Minor variations in shade, texture and natural stone/granite patterns shall not be treated as defects. |
| 13 | Construction area shall be measured and certified as mutually agreed in the final construction agreement/BOQ. |
| 14 | GST and other applicable taxes, if any, shall be charged additionally. |

15. QUOTATION VALIDITY

Validity: This quotation is subject to final site inspection and confirmation of architectural and structural drawings, final specifications and mutually agreed scope of work.

16. CLIENT / CONTRACTOR APPROVAL

| Particular | Details |
| --- | --- |
| Client Name | ____________________________________________ |
| Project / Site | ____________________________________________ |
| Construction Area | ____________________________________________ |
| Final Agreed Rate | ₹4,000/- per sq. ft. (subject to final BOQ) |
| Date | ____________________________________________ |
| Client Signature | ____________________________________________ |
| Authorized Representative | ____________________________________________ |

— END OF QUOTATION —